amba router

[specification]

RyanHunter

[Company Name]

purpose

This design is created just for creating UVM based examples, so the application of this design is non sense.

features

- support apb4 interface to receive register configurations, this block works in 200MHz.

- supports ahb5 interface to receive requests, and will be stored in a fifo.

- exec unit is configured by register unit and will fetch requests from the fifo and send out.

- support 4 output request ports by this design to select which one to deliver the stored requests

- each request port is just a simple set of self-defined, address/data interfaces

regUnit

this unit will receive apb4 write request and store the information into reg signals and giving it out to execUnit.

configurable features

- indicate the start of execUnit

- selectable priority of output

- TODO

rcvUnit

The rcvUnit is going to receive requests from an AHB5 bus, and stores them and waiting for execUnit to fetch.

execUnit

Which is an executing unit to get configurations from regUnit and going to fetch the requests from rcvUnit, and then send it to downstream ports according to an arbitrator.

- starts fetch requests after has an input start level signal (iActive).

- supports maximum 4 downstream ports

- configurable priority of each port, the larger value, the higher priority.

implementation

downstream port protocol

- Clk, the sample clock

- reqValid, the valid signal of request asserted by initiator

- reqCmd, the command signal contains address/rw/data information.

regUnit

apbIf

The apbIf module to receive and process the apb register request and translated to simple register control signals (which has VALID/ADDR/RW/WDATA/RDATA).

regBlock

A module to receive register control and give out corresponding control signals or reg read data.

rcvUnit

ahbIf

This module will process ahb requests, then converts these commands into a packet and stores into fifo. In current design, for easy development, this module support ahb write only. Every kind of read will respond an error to the bus.

fifoInst

FIFO to store commands, and wait for execUnit to fetch the commands.

execUnit